Altitude Enforcement Issues

Bruce Kavanagh

V2 9th Feb 2009

# Background

As I understand it the CIVL requirement for a 3D GPS was introduced to help with policing of both cloud flying and airspace altitude limits. More recently it has also been used for start height limits. I believe that we need to take a consistent approach on altitude verification for all three of these.

The main problem with the original proposal to use 3D GPS is that since then we have discovered that not all GPSs record the same altitude. In particular, Garmin GPSs with a pressure sensor record pressure altitude instead of GPS altitude.

Pressure altitude and GPS altitude are two fundamentally different quantities. They can be at least 600-700ft different especially in hot weather and high pressure, details of the reasons why are in Annex A. In addition the pressure altitude recorded by the Garmins with a pressure sensor is adjustable by the user at any point making it very easy for the pilot to cheat if this altitude is going to be used for altitude enforcement, as it would be almost impossible to detect changes in altitude setting if they were done in small steps or during a period of “loss of coverage” which could be pilot induced. This in my opinion makes the Garmin GPSs with a pressure sensor useless for verifying altitude. However this only applies to the Garmins with a pressure sensor and does not apply to those without a pressure sensor (which instead record GPS altitude).

Because of the potentially large difference between pressure altitude and GPS attitude it is essential that the same form of altitude is used for all competitors, it is not acceptable to use pressure altitude for some and GPS altitude for others.

The CIVL requirement is very vague just requiring a “3D GPS” and not defining what’s meant by this (though when it was written no-one was aware of the Garmin pressure sensor issue). As a result it does not provide an effective, fair means of verifying pilot altitude so we need to do something different. Some have suggested that we shouldn’t introduce any new rule on this in Cat 1 comps until it has been thoroughly tried and tested. What happened in Croatia [2006 Euro HG Cat 1] showed very clearly that the “do nothing” option definitely does not work so in my opinion we are much better going for something that we think stands the best chance of working, even if it has yet to be fully tested, rather than something that we know definitely does not work.

# What instruments record what altitude

|  | GPS Altitude | Pressure altitude 1013.25mBar | User set pressure altitude locked while recording | User set pressure altitude adjustable in flight |
| --- | --- | --- | --- | --- |
| 6030/Compeo+ | x | x |  |  |
| 5030/Compeo* | x | x |  |  |
| Brauniger IQ Competition GPS |  |  | x |  |
| Garmin without pressure sensor | x |  |  |  |
| Garmin with pressure sensor |  |  |  | x |
| MLR | x |  |  |  |
| Digifly? |  |  |  |  |
| Aircotec? |  |  |  |  |
| Flytec 4030? |  |  |  |  |
| ? |  |  |  |  |
| ? |  |  |  |  |

This list includes the main instruments but there are some gaps which need filling and there may be other instruments that pilots use which are not included.

A list of which Garmins have a pressure sensor and which don’t is included in Annex B.

The altitude recorded by Garmins with a pressure sensor can be set using auto calibrate or set to either GPS altitude, a specified altitude or based on a specified sea level pressure. This can be done at any time including during a flight.

*Although the 5030 and Compeo record GPS altitude they do not display it. Flytec re no longer supporting this model and have said that it would be a significant job to change the software to display GPS altitude

# The Options

As I see it we have the following options

## Use GPS Altitude

Any instrument that records GPS altitude would be allowed, but the Garmins with a pressure sensor would not be allowed.

Some argue that since airspace is defined in terms of pressure altitude we must check pressure altitude. However in my view there is no problem with having competition altitude limits which are different from the airspace limits so long as the competition limits are outside the airspace, in fact it could be beneficial to have some buffer between the two. The aim is to ensure that pilots stay clear of airspace, not for them to get as close to the airspace as possible.

### Advantages

- No changes are required to the current scoring software

- In all cases altitude and position data can come from a single instrument, simplifying the scoring process.

- For any pilots that don’t currently have a suitable instrument to comply with this rule it is relatively cheap to get one. The Geko 201 and Foretrex 101 or 201 are the cheapest options at around $100 reconditioned on Ebay.

- A backup instrument is cheap.

### Disadvantages

- Altitude data is not consistent with airspace limits but as described above GPS altitude limits which ensure that pilots remain clear of the (pressure altitude) airspace limits can be used so long as meet directors have an understanding of the difference between pressure altitude and GPS altitude. Generally a given GPS altitude is lower than the equivalent pressure altitude so using the airspace altitude as a GPS altitude limit would keep pilots below the airspace ceiling. In conditions with particularly low temperature or pressure upper altitude limit values might have to be reduced. Also for any lower altitude limits e.g ATZs which you need to pass over higher GPS altitude limits might need to be specified.

- Although the 5030 and Compeo currently record GPS altitude they do not display it. Flytec have indicated that it would be a significant job to add this functionality. If they don’t do that then these pilots and pilots with a pressure sensor Garmin would need to buy an instrument that records and displays GPS altitude if they don’t already have one.

## Pressure altitude from a combined instrument

i.e an instrument that records GPS position and pressure altitude.

The only instruments that I’m aware of that currently do this are the 5030/Compeo and 6030/Compeo+ (and maybe 5020 and 6020 and Brauniger equivalents, but they are not used much in hang gliding comps).

This is undoubtedly the technically best solution but will be expensive for lots of pilots that don’t already have these instruments.

### Advantages

- Greatest security

- Altitude recorded is consistent with airspace limits

- Single instrument from each pilot for download and scoring.

### Disadvantages

- Significant cost for those that don’t already have such an instrument. The 5030 is discontinued and the 6030 is $1479.

- I believe that the scoring software would need to be modified to read the pressure altitude data.

- If the pilot’s main instrument fails they are very unlikely to have a backup that also meets these requirements as these instruments are so expensive.

## Pressure altitude from a combined instrument or a separate barograph

As above but also allowing barograph instruments such as the “Brauniger IQ Competition GPS” vario to be used for altitude verification, in combination with a separate GPS for position. Garmin GPSs with a pressure sensor would not be allowed since although they record pressure altitude it is adjustable in flight.

To simplify scoring for those pilots using a separate barograph, GPS units without a pressure sensor could be used for initial screening to decide whether they got close to the altitude limit and if so the Barograph would then be downloaded to confirm the pressure altitude.

### Advantages

- Altitude recorded is consistent with airspace limits

### Disadvantages

- For pilots without a combined instrument (5030/6030 etc) they would need to provide 2 instruments for scoring if there is any doubt about altitude limits being infringed.

- Either the scoring software would need to be modified to download the barograph data from all the different barographs that pilots might use or the scorer would need stand alone software for each type of barograph.

- Pressure altitude from barographs that record the user set altitude rather than altitude for standard pressure setting would need to be corrected to standard pressure. To do this the barograph would need to be started prior to launch and the organisers would need to record the pressure altitude at launch based on standard pressure setting each day. Also the scorer would need to know which barograph instruments record user set pressure altitude and which instruments record standard pressure altitude for all the different instruments that pilots might turn in.

- The time stamps of the GPS and barograph would need to be synchronised so that the scorer could tell where the pilot was when he exceeded a particular altitude limit, i.e. was he in the area where that altitude limit applied or not.

- The two points above could potentially be achieved automatically with modifications to the scoring software, otherwise the scorer would need to do them manually which would be very time consuming. (This needs further discussion with the writer of the scoring software to see how practical it is.)

- Any pilots that don’t already have an instrument that records pressure altitude (and that excludes pressure sensor Garmins which are adjustable in flight) would need to buy one. Likely to be a minimum of $600.

## Pressure altitude from a combined instrument or separate barograph or else GPS altitude from a non-pressure sensor GPS for those without a pressure altitude recording instrument.

This is essentially Steve Kroop’s proposal but includes a distinction between the Garmins with a pressure sensor and those without. The issues are essentially as above but with the following differences.

### Advantages

- As a stop gap measure pilots without a combined instrument or barograph could use a GPS without a pressure sensor instead of buying an expensive new flight instrument.

### Disadvantages

- Pilots using a GPS for altitude verification would generally be disadvantaged as GPS altitude will generally be higher than pressure altitude but this would be an encouragement to them to get a pressure altitude recording instrument.

- If the temperature and/or pressure are particularly low or there is an altitude limit above which pilots must fly e.g. an ATZ then pilots using GPS altitude instead of pressure altitude would have an advantage and would potentially be infringing airspace even if the altitude displayed did not suggest that. Potentially different limits would need to be specified in these circumstances for those pilots who are using GPS altitude.

## Pressure altitude from a combined instrument, separate barograph or pressure sensor equipped GPS

Essentially as for option 3.3 except as described below.

### Advantages

- Pilots with a pressure sensor GPS but no barograph or combined instrument would not need to buy a new instrument.

### Disadvantages

- Pilots using pressure sensor equipped Garmin GPSs could very easily cheat by changing the altitude setting in flight.

## Altitude from any GPS unit

This is essentially the current position. Any instrument that records position and any form of altitude can be used.

### Advantages

- No change to the current instrument requirements.

- No change to the scoring software required.

### Disadvantages

- Using pressure altitude for some pilots and GPS altitude for others will give a significant advantage (potentially up to 600-700ft) to those pilots with a Garmin GPS with a pressure sensor.

- Pilots using pressure sensor equipped Garmin GPSs could very easily cheat by changing the altitude setting in flight.

## No altitude enforcement

### Advantages

- Simple

### Disadvantages

- Some pilots will break the law and cheat to gain significant advantage over others, making the competition grossly unfair and a farce if there are airspace altitude limits in the flying area.

# Where do we go from here?

There is certainly no easy answer.

My view is that GPS altitude is the way to go at least in the short term as it’s the simplest and cheapest to implement.

Using pressure altitude from combined instruments like the 6030 is undoubtedly the best way to go but I don’t believe we could require them any time soon due to the expense.

The alternative would be to use pressure altitude for verification and allow barographs to be used in addition to combined instruments. This however would require significant modification to the scoring software and/or significant extra work for the scorer. Allowing GPSs without a pressure sensor for initial sifting to decide which barographs to check would help the scorer, and using them as an interim measure for pilots without barographs or combined instruments would save them having to buy an expensive new instrument albeit giving them a slight disadvantage (however that is potentially problematic in cold temperature / low pressure and if there’s airspace that pilots must fly over as described above.)

In my view under no circumstances should we continue to allow Garmin GPS units with a pressure sensor to be used for altitude verification since it’s too easy for the pilot to adjust the altitude and cheat.

I believe that there are 3 steps that now need to be taken:

- Decide which of the options above will be used (or some other)

- Revise the rules to cover that

- Educate the pilots

The current rules in Section 7 and their interpretation are ambiguous.

16.6.1 GPS Equipment

Each pilot must ensure that he/she has equipment that is secure and compatible with the approved GPS flight verification software that is to be used. The makes and models that will be accepted for flight verification during a competition will be publicised prior to the start of the competition. In 1st Category events all pilots must, as a minimum, fly with one GPS unit capable of recording altitude as part of the track log and, from 1st May 2009, only track log evidence from 3D GPS units will be accepted as verification of a competition flight.

We now know that the Garmins with a pressure sensor are not secure, so by the letter of this rule should not be allowed, but many pilots have been using them for some time and are not aware of that. The rule states that the GPS must be capable of recording altitude, but since we now know that some GPSs record GPS altitude and some record pressure altitude this needs to be more specific so that everyone is being scored based upon the same form of altitude.

The definition of a 3D GPS is :

GPS model whose track log includes altitude coding.

which is not specific enough.

In addition there is the following rule about the onus on the pilot:

16.5.8 Airspace Infringements

Where vertical infringement of airspace or cloud is considered possible it is the responsibility of the pilot to produce track log or barogram evidence that this did not occur.

Again I believe that this is too vague. Many pilots will believe that a tracklog from a Garmin with a pressure sensor is acceptable evidence. However in my opinion since this can be adjusted in flight it does not really prove anything and should not be considered acceptable evidence under this rule. A secure barograph trace or pressure altitude trace from a combined instrument (6030etc) would be acceptable. A GPS altitude trace would potentially also be acceptable if for the atmospheric condition of the day that GPS altitude could be shown beyond reasonable doubt to be outside the airspace altitude limit.

Any new rule would also need careful thought about how to handle any breaks in the altitude trace to ensure that pilots could not just switch the instrument off while they exceed the altitude limit while also not penalising pilots who had a genuine loss of GPS reception.

Likewise if there is a break in a barograph trace a pilot could potentially adjust the altitude setting before restarting the barograph, but we wouldn’t want to penalise a pilot who had a genuine instrument malfunction and had to restart it. A GPS altitude trace (from a GPS without a pressure sensor) to go with the barograph would clearly show if the barograph pressure setting had been adjusted when it was restarted.

# Annex A

This is copy of an article I wrote for the BHPA magazine “Skywings” that explains how and why GPS and pressure altitude can be so different.

Differences between Pressure and GPS Altitude

Using GPS to police altitude limits has some potential pitfalls for competitors and organisers. The height recorded by a GPS can be up to 10% higher than the pressure altitude shown on an altimeter. So a GPS may suggest that a pilot has infringed an airspace ceiling when actually he hasn’t.

Here's the background as to why.

In this discussion I'll ignore instruments that have both a pressure sensor and a GPS receiver. When I say altimeter I mean a simple instrument without a GSP e.g. a Brauniger IQ comp and when a say GPS I mean one with a height output but without a pressure sensor e.g. an MLR.

I'll start with a few definitions.

Geometric height is the vertical distance between where you are and the sea level datum, as you would measure using a very long tape measure. This is essentially the height data that is output and recorded by a GPS, however the GPS will use the WGS84 ellipsoid as its zero height datum rather than sea level (more on that later).

ICAO define a international standard atmosphere (ISA) with a pressure at sea level of 1013.2mBar and lapse rate (decrease in temperature with increasing altitude) of 6.5degC/1000m. Knowing these two values, the properties of air and the acceleration due to gravity, it is possible to calculate the pressure at any geometric height above sea level. This is shown as the blue line in figure 1.

Figure 1 ICAO Standard Atmosphere

This standard relationship between pressure and height is used to define pressure altitude. To cope with non-standard pressures the sea level pressure setting in the altimeter can be adjusted so that the altimeter still reads zero at sea level which is equivalent to moving the line in figure 1 up or down to give the correct sea level pressure. However, this only corrects for variations in pressure not temperature so the altimeter still reads a form of pressure altitude, not geometric height.

Geometric height is only used on air maps to define the height of terrain and is not used for any airspace. All airspace is defined in terms of pressure altitude either based on the standard sea level pressure (1013.2mBar) or the actual sea level pressure.

The first difference between pressure altitude and GPS height comes about because the WGS 84 ellipsoid which defines zero GPS height can be up to 100m from the actual sea level, which is used as the reference for pressure altitude. In Europe the WGS84 ellipsoid is around 30m above sea level.

There are further small differences due to errors in each instrument, particularly the GPS for which the altitude measurement is significantly less accurate than the position measurement. GPS height errors will typically be less than 30-50m but can be more especially transiently if signal quality is poor. Altimeter errors will typically be less than 10m but can be up to around 30m and may degrade with age if not recalibrated.

However, the largest differences come about because the change in pressure with geometric height depends upon the air density which in turn depends upon temperature as well as pressure.

If you imagine a 1m cube of still air, it has 3 vertical forces acting on it which must balance (figure 2).

Figure 2 Forces on a Cube of Air

1. A downwards force on the top face equal to the air pressure there (Pt) x the area (1m2) so the force is Pt

2. A downwards gravitational force due to the weight of the air in the cube = density x volume (1m3) x g (acceleration due to gravity)

3. An upwards force on the bottom face equal to the air pressure there (Pb) x the area (1m2) so the force is Pb

So Pt +(density x g) = Pb

So the change in pressure per meter of geometric height = Pt-Pb = -density x g

What this means is that on a hot day when the air density is lower, the change in pressure (and hence by definition the change in pressure altitude) with increasing geometric height is less than on a "standard" ISA day. The pink line on figure 1 shows the relationship between pressure and geometric height if the air temperature at all altitudes is 25 deg C hotter than the ISA temperature for the same pressure altitude, ie 40deg C at sea level, and around 31deg C at around 5000ft, typical of a fairly hot continental flying day. For this idealised hot atmospheric temperature profile the geometric height (as measured by a GPS) is about 9% higher than the pressure altitude as measured by an altimeter set to the correct sea level pressure.

Figure 3 shows a GPS and altimeter trace from a warm but not excessively hot European day. The pilot has set his altimeter to match GPS altitude at launch. As he climbs above that altitude the GPS progressively reads higher than the altimeter and vice versa as he descends below launch altitude. Figure 4 shows the difference between the two measurements plotted against the GPS height. In addition to the general trend there are other random errors from one or both instruments but these are generally less than 20m.

Figure 3 Typical GPS & Altimeter Trace

Figure 4 Differences Between GPS Height and Pressure Altitude

So since a GPS and a pressure altimeter are measuring two fundamentally different quantities, even if they are both 100% accurate they will generally not read the same altitude. The pressure altimeter is measuring pressure and representing that as an altitude based on some essentially arbitrary assumptions about the "standard" atmosphere. Comparing the two is a bit like comparing the straight line distance between two points from a GPS with the trip milometer distance from driving along a winding road between them. Although the two distances are related they will not be the same even if both instruments are 100% accurate, except in the special case of a straight road. Likewise the GPS and pressure altimeter will only read the same altitude if the temperature profile in the atmosphere matches the profile assumed in the standard atmosphere, and the pressure altimeter has been corrected for the actual sea level pressure.

Most electronic varios have temperature compensated pressure sensors but all that does is ensure that the sensor reads the correct pressure as temperature varies. It does not in any way try to compensate for the effect of temperature on the relationship between pressure altitude and geometric height. To do that you'd need to know the complete temperature profile between sea level and your altitude so it is totally impractical.

The key issues for competition flying are as follows.

1. In hot places GPS height can be up to 10% greater than pressure altitude after accounting for variations in sea level pressure so a GPS will tend to show that you’ve busted an altitude ceiling before you actually have. There will be further differences due to errors in each instrument.

2. In Europe this will be partly offset by the fact that the WGS84 ellipsoid is around 30m above sea level.

3. Setting an altimeter by setting it to the geometric height at launch rather than setting it to zero at sea level (or setting the sea level pressure) can result in an error of up to 10% of the launch height.

4. Competition organisers need to understand the differences between pressure altitude and geometric height and must be very clear in briefings about whether any altitude limits that they specify are geometric heights or pressure altitudes and how they will be enforced. There's a bit of a mismatch here as all airspace limits are pressure altitudes but the proposal is to use GPS (Geometric) height data to enforce those limits.

5. For instruments with a GPS receiver and pressure sensor, competition organisers must understand which altitude is being recorded by the instrument so that altitude limits can be enforced in a consistent way.

# Annex B – Garmins with and without a pressure sensor

Without sensor
Geko 201
Foretrex 101
Foretrex 201
eTrex Legend
eTrex Legend HCx
eTrex Venture HC
GPS 60 
GPS 72
GPS 76
GPSMAP 60
GPSMAP 60Cx
GPSMAP 76
GPSMAP 76 Cx
GPSMAP 96
GPSMAP 96C
Oregon 200

Discontinued Without Sensor
eTrex
eTrex Legend Cx
eTrex Venture
eTrex Venture Cx
GPSMAP 60C
GPSMAP 76C

With Sensor
Geko 301
eTrex Summit HC
eTrex Vista

eTrex Vista H
eTrex Vista HCx
GPSMAP 60CSx
GPSMAP 76S
GPSMAP 76 CSx
Colorado 300

Colorado 400c

Colorado 400i

Colorado 400t
Oregon 300
Oregon 400c

Oregon 400i

Oregon 400t

Discontinued With Sensor
eTrex Summit
eTrex Vista Cx
GPSMAP 60CS
GPSMAP 76CS